Restraint for existing mechanical joints found on ductile iron pipes shall consist of the following: The restraint shall be manufactured of ductile iron conforming to ASTM A536. The restraint devices shall be coated using MEGA-BOND. (For complete specifications on MEGA-BOND visit www.ebaa.com.) The split restraint ring, incorporating a plurality of individually-actuating gripping surfaces, shall be used to grip the pipe, and a sufficient number of bolts shall be used to connect the restraint to the mechanical joint. The combination shall have a minimum working pressure rating as shown in the adjacent table. The restraint shall be the Series 1100SD, as manufactured by EBAA Iron, Inc., or approved equal.