Sample Specification 2500, 4 inch through 12 inch

Restraint for PVC pipe (AWWA C900-16) at PVC fittings shall consist of the following: The restraint shall be manufactured of ductile iron conforming to ASTM A536. The restraint devices shall be coated using MEGA-BOND™. (For complete specifications on MEGA-BOND visit www.ebaa.com.) Side clamp bolts shall be of SAE J429 Grade 5 material. A split ring shall be utilized on the PVC fitting bell. A serrated ring shall be used to grip the spigot end of the pipe, and a sufficient number of bolts shall be used to connect the bell ring and the gripping ring. The combination shall have a minimum working pressure rating equivalent to the pipe. The restraint shall be the Series 2500, as manufactured by EBAA Iron, Inc., or approved equal.

Sample Specification 2500, 14 inch through 48 inch

Restraint for PVC pipe (AWWA C900-16 & CI O.D.) at PVC fittings shall consist of the following: The restraint shall be manufactured of ductile iron conforming to ASTM A536. Side clamp bolts shall be of SAE J429 Grade 5 material. A split ring shall be utilized on the PVC fitting bell. A restraint ring, incorporating a plurality of individually-actuating gripping surfaces, shall be used to grip the pipe, and a sufficient number of bolts shall be used to connect the bell ring and the gripping ring. The combination shall have a minimum working pressure rating equivalent to the pipe. The restraint devices shall be coated using MEGABOND™. (For complete specifications on MEGABOND visit www.ebaa.com). The restraint shall be the Series 2500, as manufactured by EBAA Iron, Inc., or approved equal.