Sample Spec 2500 4 inch through 12 inch

Restraint for PVC pipe (AWWA C900) at PVC fittings shall consist of the following: The restraint shall be manufactured of ductile iron conforming to ASTM A536. The restraint devices shall be coated using MEGA-BOND™. (For complete specifications on MEGA-BOND visit www.ebaa.com.) A split ring shall be utilized on the PVC fitting bell. A serrated ring shall be used to grip the pipe, and a sufficient number of bolts shall be used to connect the bell ring and the gripping ring. The combination shall have a minimum working pressure rating equivalent to the pipe. The restraint shall be the Series 2500, as manufactured by EBAA Iron, Inc., or approved equal.

Sample Spec 2500 14 inch through 36 inch

Restraint for PVC pipe (AWWA C905 & CI O.D.) at PVC fittings shall consist of the following: The restraint shall be manufactured of ductile iron conforming to ASTM A536. A split ring shall be utilized on the PVC fitting bell. A restraint ring, incorporating a plurality of individually-actuating gripping surfaces, shall be used to grip the pipe, and a sufficient number of bolts shall be used to connect the bell ring and the gripping ring. The combination shall have a minimum working pressure rating equivalent to the pipe. The restraint devices shall be coated using MEGABOND™. (For complete specifications on MEGABOND visit www.ebaa.com). The restraint shall be the Series 2500, as manufactured by EBAA Iron, Inc., or approved equal.

For Sizes 42 inch and 48 inch See Specifications for Series 2200 Brochure