Restraint for AWWA C900 or C909 PVC Pipe at PVC Fittings shall consist of the following: The restraint device shall be manufactured of ductile iron conforming to ASTM A536, and shall be coated with MEGA-BOND. (For complete specifications on MEGA-BOND visit www.ebaa.com.) Side clamp bolts shall be of SAE J429 Grade 5 material. A split serrated ring shall be used on the plain end or spigot end of the PVC pipe. A split non-serrated ring shall go behind the gasket race of the PVC fitting. The two rings shall be harnessed together by an array of thrust rods. The combination of the restraint(s), thrust rods, and fasteners shall have a pressure rating listed in table 1.1 of the current product brochure. The restraint shall have a minimum two to one safety factor. The restraint device shall be the Series 2600, as manufactured by EBAA Iron, Inc., or approved equal.